SYMULACJA:

KOD:

/**************************************/

/*              ARE 2008              */

/*      e-mail: biuro@are.net.pl      */

/*      www   : are.net.pl            */

/**************************************/

// Jan Bronicki 249011

// Borys Staszczak 248958

#define __AVR_ATmega32__

#define F_CPU 8000000UL

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <string.h>

void delay_ms(int ms)

{

volatile long unsigned int i;

for (i = 0; i < ms; i++)

_delay_ms(1);

}

void delay_us(int us)

{

volatile long unsigned int i;

for (i = 0; i < us; i++)

_delay_us(1);

}

#define RS 0

#define RW 1

#define E 2

void LCD2x16_init(void)

{

PORTC &= ~(1 << RS);

PORTC &= ~(1 << RW);

PORTC |= (1 << E);

PORTD = 0x38; // dwie linie, 5x7 punktow

PORTC &= ~(1 << E);

_delay_us(120);

PORTC |= (1 << E);

PORTD = 0x0e; // wlacz wyswietlacz, kursor, miganie

PORTC &= ~(1 << E);

_delay_us(120);

PORTC |= (1 << E);

PORTD = 0x06;

PORTC &= ~(1 << E);

_delay_us(120);

}

void LCD2x16_clear(void)

{

PORTC &= ~(1 << RS);

PORTC &= ~(1 << RW);

PORTC |= (1 << E);

PORTD = 0x01;

PORTC &= ~(1 << E);

delay_ms(120);

}

void LCD2x16_putchar(int data)

{

PORTC |= (1 << RS);

PORTC &= ~(1 << RW);

PORTC |= (1 << E);

PORTD = data;

PORTC &= ~(1 << E);

_delay_us(120);

}

void LCD2x16_pos(int wiersz, int kolumna)

{

PORTC &= ~(1 << RS);

PORTC &= ~(1 << RW);

PORTC |= (1 << E);

delay_ms(1);

PORTD = 0x80 + (wiersz - 1) * 0x40 + (kolumna - 1);

delay_ms(1);

PORTC &= ~(1 << E);

_delay_us(120);

}

// Set point (in 0.1%)

int _sp = 400;

// Histereza (in 0.1%)

int _h = 80;

// Nieczułość (in 0.1%)

int _n = 160;

// Error value

int _e;

// Integer part of the error

int int_e;

// Decimal value of the error

int dec_e;

// Whole process value (in 0-1023 range)

float process_value;

// Process value with decimal part

int _pv;

// Integer part of process value

int _ipv;

// Decimal part of process value

int _decpv;

int main(void)

{

char tmp[16];

int i;

DDRD = 0xff;

PORTD = 0x00;

DDRC = 0xff;

PORTC = 0x00;

DDRB = 0x00;

PORTB = 0xff;

_delay_ms(500);

LCD2x16_init();

LCD2x16_clear();

ADMUX = 0x40;

ADCSRA = 0xe0;

while (1)

{

// Start an ADC conversion by setting ADSC bit (bit 6)

ADCSRA = ADCSRA | (1 << ADSC);

// Wait until the ADSC bit has been cleared

while (ADCSRA & (1 << ADSC))

;

//_n=_h+_h;

process_value = ADC;

_pv = (process_value / 1023.0) * 1000;

_ipv = _pv / 10;

_decpv = _pv % 10;

_e = _sp - _pv;

int_e = _e / 10;

dec_e = _e % 10;

// LED CV1 ON

if (_e > ((_n/2)+_h))

{

PORTC = ~(0x01 << 4);

}

// LED OFF

if(_e < _n/2 && _e > -_n/2)

{

PORTC=(0x00);

}

// LED CV2 ON

if (_e < ((-_n/2)-_h))

{

PORTC = ~(0x01 << 3);

}

if (!(PINB & (8 << PB0)))

{

_sp = 50;

}

if (!(PINB & (4 << PB0)))

{

_sp = 40;

}

if (!(PINB & (2 << PB0)))

{

_h = 8;

_n = 16;

}

if (!(PINB & (1 << PB0)))

{

_h = 10;

_n = 20;

}

LCD2x16_pos(1, 1);

sprintf(tmp, "SP=%2d PV=%3d.%1d%% ", _sp/10, _ipv, abs(_decpv));

for (i = 0; i < 16; i++)

{

LCD2x16_putchar(tmp[i]);

}

LCD2x16_pos(2, 1);

sprintf(tmp, "H=%2d   E=%3d.%1d%% ", _h/10, int_e, abs(dec_e));

for (i = 0; i < 16; i++)

{

LCD2x16_putchar(tmp[i]);

}

delay_ms(500);

}

return 0;

}